Model:UT-6616M-I

10/100M TCP/IP to 16 Ports RS-485/422 Serial Device Serve

with Opto-Electronic Isolation

- Datasheet

UTEK TECHNOLOGY (SHENZHEN) CO., LTD.

Add: Floor 8-10, Building 7, Skyworth Innovation Valley, No. 8, Tangtou No.1 Road, Shiyan Street, Shiyan Old Street, Bao 'an District, Shenzhen

Tel：+86-755-81202008

Fax：+86-755-27886083

Http：www.uotek.com

- Overview

UT-6616M-I series serial device server, is a converter between asynchronous serial port RS-422/485 and Ethernet. It is a standalone intelligent device with CPU and embedded OS and full TCP/IP protocol stack. It completes transparent data transfer between RS-422/485 and Ethernet in both directions, allowing RS-422/485 serial devices to connect to the network immediately.

- Technical Parameters

Hardware

●  UNIX Fixedtty mechanism

●  With 16 serial ports, it can be connected to various serial devices such as terminals, Modems, barcode machines, cash registers, ISDN, terminal adapters, serial printers and PCs.

●  With Reset button

●  Ethernet port 10/100M transmission rate, auto negotiation

●  Provides with full signal: TR+, TR-, RXD+, RXD-, GND.

●  32 bits embedded 100M CPU, 8M RAM

●  LED indication for working status

Software

●  Support ARP, IP, ICMP, UDP, TCP, PPP, HTTP, TELNET, DNS, DHCP, PAP, CHAP.
●  Support Windows extended Serial Mode
●  Support UNIX host login mode
●  Support DNS
●  Support ARP
●  Terminal server parameters configuration
●  Support for status monitoring
●  Support MODEM connection

| Product model |  | UT-6616M-I |  |
| --- | --- | --- | --- |
| serial port number |  | 16 |  |
| CPU |  | 32bit 100MHZ |  |
| RAM |  | 8M |  |
| Serial port | Baud rate | 50-460800bps |  |
|  | Check bit | Nove,Odd, Even, Mark, Space |  |
|  | Data bit | 5,6,7,8 |  |
|  | Stop bit | 1, 1.5, 2 |  |
|  | Flow control | RTS/ CTS, XON/ XOFF |  |
|  | Form | Binding post |  |
|  | Serial port protection | 3KV lightning and surge protection | ±15KV Anti-static protection |
|  | Isolation protection | Isolation voltage 2500V rms |  |
|  | Signal | RS422:TXD+/RXD+/TXD-/RXD-/GND, RS485:Data+/Data- |  |
| Ethernet port | Rate | Auto-10/100M |  |
|  | Form | RJ45 |  |
|  | Protection | 3KV lightning and surge protection |  |
| Software | Protocol | ARP, IP, ICMP, UDP, TCP, PPP, HTTP, TELENT, DNS, DHCP, PAP, CHAP |  |
|  | Virtual COM | Window NT/2000/XP |  |
| Environment | Operating temperature | -10 ~60℃ |  |
|  | Operating humidity | 5 ~ 95% |  |
|  | Storage temperature | -20 ~ 85℃ |  |
|  | Storage humidity | 5 ~ 95% |  |
| Certification |  | FCC, CE |  |
| Power |  | AC 220V |  |
| Appearance | Material | Metal housing |  |
|  | Dimension | 19 inches, IU height |  |

- Pin definition

Front side of terminal

RS-422 full duplex connection

RS-485 half-duplex connection

- Product View (Appearance)

- Structure Dimensions

- Ordering Information

| Model | Signal/ Interface |  | Port number |  | Interface protection level |  | Operating Environment |  |  | Power |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TCP/IP | RS-485/422 |  |  |  |  | Temperature |  | Humidity |  |  |
|  | RJ45 | RJ45 | TCP/IP | RS-485/422 | TCP/IP | RS-485/422 | -10~60℃ | -40~85℃ | 5~95% | plug and play | External power |
| UT-6616M-I | √ | √ | 1 | 16 | 3KV lightning protection |  | √ |  | √ |  | 220VAC |